Development and Known-Truth Validation of a Two-Input Transfer Function Analysis Pipeline for Cerebral Hemodynamics

Abstract

Transfer Function Analysis (TFA) of dynamic cerebral autoregulation (dCA) generally treats mean arterial pressure (MAP) as a single input affecting the output of cerebral blood flow velocity (CBFV). As such, the system describing dCA is modeled as a single-input-single-output (SISO) system that provides the complex relationship, including gain and phase, between MAP and CBFV. However, previous research has shown that other inputs, such as end-tidal carbon dioxide (PETCO₂), can also influence CBFV. Therefore, it is important to evaluate a multiple-input-single-output (MISO) model that accounts for the relationships between both MAP and PETCO₂ and the output of CBFV. Furthermore, a MISO model can use the shared frequency content between MAP and PETCO₂ when estimating each input-to-output relationship, as opposed to two separate SISO models that relate either MAP or PETCO₂ to CBFV without accounting for the other input. While previous studies have shown higher multiple coherence with multiple-input models, this study seeks to determine whether that increased coherence corresponds to improved pathway accuracy or instead reflects the greater flexibility of a model with an additional input. In this study, we develop and validate a transparent MISO TFA pipeline that jointly estimates the MAP-to-CBFV and PETCO₂-to-CBFV transfer functions. The pipeline provides a consistent procedure for spectral estimation, the MISO solution, gain and phase calculation, model diagnostics, statistical comparison, and reporting. Short recordings, weak PETCO₂ fluctuations, correlated inputs, measurement noise, timing errors, and poorly conditioned spectral matrices may offset the conceptual benefit of separating the two pathways. The pipeline is therefore evaluated using a known-truth simulation framework to identify the conditions under which two-input MISO transfer function analysis recovers the true MAP and PETCO₂ pathways more accurately than separate SISO models. Each simulated family represents a fixed physiological system, while recording duration, noise, timing alignment, and estimator settings are varied as observation conditions without changing the underlying truth. Accuracy will be evaluated using frequency-resolved gain error, wrapped phase error, normalized complex error, and a signed model-advantage measure that indicates whether MISO or SISO has lower error. Resting baseline recordings from cognitively normal (NC) adults will then be placed within the simulated operating space to determine whether their duration, input relationships, excitation, and conditioning support interpretable MISO estimates. The purpose of this paper is to develop a reproducible MISO TFA pipeline, explain the reasoning behind its major analytical choices, establish the conditions under which the additional input is justified, determine when SISO remains sufficient, and identify when neither separated pathway should be interpreted without additional data.

Introduction

The brain requires continuous blood flow even as arterial pressure changes from beat to beat. Cerebral autoregulation describes the physiological processes that limit how strongly these pressure changes alter cerebral perfusion (Regulation of Cerebral Blood Flow in Humans). Dynamic cerebral autoregulation focuses on responses that occur over seconds to minutes. TFA is commonly used to study these responses by treating MAP as the input and CBFV as the output (Transfer Function Analysis of Dynamic Cerebral Autoregulation). The resulting transfer function is complex-valued. Gain describes how strongly changes in MAP appear in CBFV, while phase describes their timing relationship at each frequency. Coherence is also used to describe how consistently the two signals are linearly related.

A MAP-only model is useful, but MAP is not the only input that can influence CBFV. Carbon dioxide, being a potent vasodilator, has a strong effect on cerebrovascular tone, and PETCO₂ provides a practical continuous estimate of carbon dioxide during resting recordings (Regulation of the Cerebral Circulation by Arterial Carbon Dioxide). When MAP and PETCO₂ share frequency content, a MAP-only SISO estimate can include part of the PETCO₂-related response. The same issue applies to a PETCO₂-only SISO estimate. Therefore, two separate SISO models describe pairwise relationships, but they do not separate the contribution of one measured input from the other.

Previous studies have used multivariable models to evaluate MAP and carbon dioxide together (Multivariate Dynamic Analysis of Cerebral Blood Flow Regulation; Multivariate System Identification for Cerebral Autoregulation). These studies provide a strong basis for using a two-input model. However, the use of MISO TFA is not yet organized into one clear and reproducible workflow for resting cerebral hemodynamic recordings. Studies may use different preprocessing choices, spectral settings, model solutions, diagnostic criteria, and reporting methods. In addition, higher multiple coherence shows that the model fits the measured output more closely, but it does not show that the separated MAP and PETCO₂ pathways are more accurate.

[Figure 1 placeholder: Comparison of SISO and MISO model structure]

Figure note: Show two separate SISO models beside one joint MISO model. The figure should make clear that the MISO solution estimates the MAP and PETCO₂ transfer functions together while accounting for the relationship between the two inputs.

The first goal of this paper is therefore to develop and document a complete MISO TFA pipeline. The pipeline begins with MAP, PETCO₂, and CBFV signals and applies the same preprocessing and spectral estimation procedure to each recording. The MAP and PETCO₂ auto-spectra, their cross-spectrum, and their cross-spectra with CBFV are then used in one joint MISO solution. The pipeline returns MAP and PETCO₂ gain and phase, coherence measures, input power, and conditioning diagnostics. It also provides the statistical comparisons, figures, tables, and source data needed to understand and reproduce the analysis. Each major choice will be explained so that the workflow can be inspected and modified rather than treated as an ambiguous software procedure.

[Figure 2 placeholder: Proposed MISO TFA analysis pipeline]

Figure note: Show the flow from MAP, PETCO₂, and CBFV signals through preprocessing, Welch spectral estimation, the joint MISO solution, gain and phase calculation, conditioning and identifiability checks, statistical analysis, and organized figures and source-data exports.

Adding a second input does not automatically produce a more accurate model. MISO must estimate two pathway coefficients from the same finite recording. If MAP and PETCO₂ become too similar within a frequency region, the input spectral matrix can become poorly conditioned and small spectral errors can produce large changes in the separated transfer functions. Short recordings, weak PETCO₂ fluctuations, measurement noise, signal misalignment, and estimator settings may also affect this balance. A useful pipeline must therefore report the conditions that support the MISO estimates, not only the estimates themselves.

Subject recordings cannot show the true MAP-to-CBFV or PETCO₂-to-CBFV transfer functions. If MISO and SISO produce different results in an observed recording, that difference alone cannot show which model is closer to the underlying physiology. A known-truth simulation makes this comparison possible. MAP and PETCO₂ inputs can be generated with assigned relationships, passed through predefined transfer functions, and combined into a CBFV output. MISO and SISO can then be compared with the same known pathways while recording duration, noise, input relationships, timing, and estimator settings are varied.

This paper therefore has three connected objectives. The first is to develop and clearly document the MISO TFA pipeline and the reasoning behind its main choices. The second is to validate the pipeline using known-truth simulations and determine when MISO or SISO has lower pathway error. The third is to place NC recordings within the simulated operating space and determine whether the observed data support interpretable MISO estimates. The goal is to propose a reproducible reference workflow that can support future research.

This part is just for reference

Study Overview

This study has three connected parts. First, the MISO TFA pipeline is developed and documented from signal preprocessing through spectral estimation, the joint MISO solution, model diagnostics, statistical testing, and organized outputs. Second, known-truth simulations are used to determine when MISO or SISO more accurately recovers the MAP and PETCO₂ pathways as the physiological system and observation conditions change. Third, NC recordings are placed within the simulated operating space to determine whether their duration, input relationships, excitation, and conditioning support interpretable MISO estimates.

Model performance is summarized using gain error, wrapped phase error, normalized complex error, and model advantage. Positive model advantage favors MISO, while negative model advantage favors SISO. Statistical significance will always be interpreted together with this direction and the size of the difference.

Reference Papers Organized by Purpose

This working library includes every source cited in either the current approach-paper draft or the earlier combined MISO and MCI draft. Each paper title is linked to an online paper record or full-text page so that the source can be opened and downloaded.

Cerebral Blood Flow, dCA, and TFA Foundations

Use in the papers: These sources establish the physiology of cerebral blood flow regulation, the accepted definition and reporting of TFA, related methodological recommendations, and the basis for measuring CBFV with transcranial Doppler ultrasound.

Claassen JAHR, Thijssen DHJ, Panerai RB, Faraci FM. Regulation of cerebral blood flow in humans: physiology and clinical implications of autoregulation. Physiological Reviews. 2021;101(4):1487-1559. doi:10.1152/physrev.00022.2020

Panerai RB, Brassard P, Burma JS, et al.; Cerebrovascular Research Network. Transfer function analysis of dynamic cerebral autoregulation: A CARNet white paper 2022 update. Journal of Cerebral Blood Flow & Metabolism. 2023;43(1):3-25. doi:10.1177/0271678X221119760

Kostoglou K, Bello-Robles F, Brassard P, et al. Time-domain methods for quantifying dynamic cerebral blood flow autoregulation: Review and recommendations. A white paper from the Cerebrovascular Research Network (CARNet). Journal of Cerebral Blood Flow & Metabolism. 2024;44(9):1480-1514. doi:10.1177/0271678X241249276

Aaslid R, Markwalder TM, Nornes H. Noninvasive transcranial Doppler ultrasound recording of flow velocity in basal cerebral arteries. Journal of Neurosurgery. 1982;57(6):769-774. doi:10.3171/jns.1982.57.6.0769

Carbon Dioxide and Multivariable Cerebral Hemodynamic Modeling

Use in the papers: These papers support PETCO₂ as a meaningful cerebrovascular input and provide the direct basis for estimating MAP, PETCO₂, and CBFV together rather than as isolated signal pairs.

Hoiland RL, Fisher JA, Ainslie PN. Regulation of the cerebral circulation by arterial carbon dioxide. Comprehensive Physiology. 2019;9(3):1101-1154. doi:10.1002/cphy.c180021

Panerai RB, Simpson DM, Deverson ST, Mahony P, Hayes P, Evans DH. Multivariate dynamic analysis of cerebral blood flow regulation in humans. IEEE Transactions on Biomedical Engineering. 2000;47(3):419-423. doi:10.1109/10.827312

Peng T, Rowley AB, Ainslie PN, Poulin MJ, Payne SJ. Multivariate system identification for cerebral autoregulation. Annals of Biomedical Engineering. 2008;36(2):308-320. doi:10.1007/s10439-007-9412-9

MCI, Alzheimer's Disease, and Biological-Marker Background

Use in the papers: These sources mainly support the earlier combined draft and the separate biological-marker paper. They describe cerebral hemodynamic findings in MCI and Alzheimer's disease and provide examples of model-based physiomarkers and carbon-dioxide-sensitive approaches.

Kisler K, Nelson AR, Montagne A, Zlokovic BV. Cerebral blood flow regulation and neurovascular dysfunction in Alzheimer disease. Nature Reviews Neuroscience. 2017;18(7):419-434. doi:10.1038/nrn.2017.48

Beishon L, Haunton VJ, Panerai RB, Robinson TG. Cerebral hemodynamics in mild cognitive impairment: A systematic review. Journal of Alzheimer's Disease. 2017;59(1):369-385. doi:10.3233/JAD-170181

Gommer ED, Martens EGHJ, Aalten P, et al. Dynamic cerebral autoregulation in subjects with Alzheimer's disease, mild cognitive impairment, and controls: Evidence for increased peripheral vascular resistance with possible predictive value. Journal of Alzheimer's Disease. 2012;30(4):805-813. doi:10.3233/JAD-2012-111628

Tarumi T, Dunsky DI, Khan MA, et al. Dynamic cerebral autoregulation and tissue oxygenation in amnestic mild cognitive impairment. Journal of Alzheimer's Disease. 2014;41(3):765-778. doi:10.3233/JAD-132018

Heutz R, Claassen J, Feiner S, et al. Dynamic cerebral autoregulation in Alzheimer's disease and mild cognitive impairment: A systematic review. Journal of Cerebral Blood Flow & Metabolism. 2023;43(8):1223-1236. doi:10.1177/0271678X231173449

Marmarelis VZ, Shin DC, Orme ME, Zhang R. Model-based physiomarkers of cerebral hemodynamics in patients with mild cognitive impairment. Medical Engineering & Physics. 2014;36(5):628-637. doi:10.1016/j.medengphy.2014.02.025

Marmarelis VZ, Shin DC, Tarumi T, Zhang R. Comparison of model-based indices of cerebral autoregulation and vasomotor reactivity using transcranial Doppler versus near-infrared spectroscopy in patients with amnestic mild cognitive impairment. Journal of Alzheimer's Disease. 2017;56(1):89-105. doi:10.3233/JAD-161004

Hashem S, Yamashiro S, Joe E, Chui H, Marmarelis V. PRBS gas challenges reveal impaired chemoreflex and cholinergic dynamics in MCI. Annals of Biomedical Engineering. 2026;54(8):2497-2507. doi:10.1007/s10439-026-04213-7

Spectral Estimation and Statistical Methods

Use in the papers: These sources support Welch spectral estimation, circular treatment of phase, unequal-variance testing, standardized effect size, and false-discovery-rate control across repeated frequency-bin comparisons.

Welch PD. The use of fast Fourier transform for the estimation of power spectra: A method based on time averaging over short, modified periodograms. IEEE Transactions on Audio and Electroacoustics. 1967;15(2):70-73. doi:10.1109/TAU.1967.1161901

Mardia KV, Jupp PE. Directional Statistics. Chichester, UK: Wiley; 2000. doi:10.1002/9780470316979

Berens P. CircStat: A MATLAB toolbox for circular statistics. Journal of Statistical Software. 2009;31(10):1-21. doi:10.18637/jss.v031.i10

Welch BL. The generalization of Student's problem when several different population variances are involved. Biometrika. 1947;34(1-2):28-35. doi:10.1093/biomet/34.1-2.28

Hedges LV. Distribution theory for Glass's estimator of effect size and related estimators. Journal of Educational Statistics. 1981;6(2):107-128. doi:10.2307/1164588

Benjamini Y, Hochberg Y. Controlling the false discovery rate: A practical and powerful approach to multiple testing. Journal of the Royal Statistical Society Series B. 1995;57(1):289-300. doi:10.1111/j.2517-6161.1995.tb02031.x

Cognitive Assessment Methods from the Earlier Combined Draft

Use in the papers: These papers define the MMSE and MoCA measures used to characterize cognitive status in the earlier clinical draft. They are not central to the approach-paper introduction but remain part of the complete working reference library.

Folstein MF, Folstein SE, McHugh PR. “Mini-mental state”: A practical method for grading the cognitive state of patients for the clinician. Journal of Psychiatric Research. 1975;12(3):189-198. doi:10.1016/0022-3956(75)90026-6

Nasreddine ZS, Phillips NA, Bédirian V, et al. The Montreal Cognitive Assessment, MoCA: A brief screening tool for mild cognitive impairment. Journal of the American Geriatrics Society. 2005;53(4):695-699. doi:10.1111/j.1532-5415.2005.53221.x